UGVCL Smart Meter Live Monitor

Detailed Technical Documentation

Architecture • Databases • Data Flow • Deployment

| Document Version | 1.0 |
| --- | --- |
| Project Name | ugvcl-live-dashboard |
| Prepared For | UGVCL Operations & Support Team |
| Environment | UGVCL Internal Network (On-Premise) |
| Technology | Python 3.8+ (Standard Library Only), HTML/CSS/Vanilla JS |
| Status | Ready for Purpose |

# Table of Contents

1. Objective

2. Scope

3. Abbreviations

4. Overview

5. High-Level Architecture

6. Component Architecture

7. Database Architecture (Complete)

8. End-to-End Data Flow

9. Detailed Application Flow

10. API Reference

11. Caching, Resiliency & Performance

12. Frontend Design

13. Configuration Reference (config.json)

14. Deployment Guide

15. Diagnostics & Troubleshooting

16. Security Considerations

17. Ready-To-Purpose Checklist

18. Appendix

# 1. Objective

The UGVCL Smart Meter Live Monitor is a purpose-built, read-only dashboard that consolidates multiple UGVCL Grafana smart meter reports into a single unified view. The primary objective is to provide near real-time visibility of:

Meter data reception (Midnight, Load Profile, Instant, Billing)

Communication status (RF vs GPRS)

Device inventory & metering modes

Bulk "Last Communication" verification for meter serials

Historical trend analysis for any date up to 1 year

# 2. Scope

Aggregate data across 4 CLRs (CLR1, CLR2, CLR3, CLR4)

Replicate logic from existing UGVCL Grafana dashboards

Operate without pip, npm or external internet access (chart library bundled)

Act as a secure caching layer between browsers and Grafana

Provide resilience against partial query failures

Support LAN sharing via --host 0.0.0.0

Support offline/demo validation via --demo

# 3. Abbreviations

| Abbreviation | Meaning |
| --- | --- |
| CLR | Circle Load Reporter |
| MPRT | MeasPointResType (maps MeasPoint to ResultType) |
| RF | Radio Frequency communication (ConnectUsing = NetworkUdp) |
| GPRS | General Packet Radio Service (meters not using NetworkUdp) |
| LP | Load Profile (result_30m_double) |
| MSSQL | Microsoft SQL Server (Device Master & Attribute Store) |
| PostgreSQL / PG | PostgreSQL (Measurement Result Store) |
| UID | Unique Identifier for a Grafana datasource |
| AttrKey / AttrVal | Attribute key/value metadata of a meter |

# 4. Overview

This application does not connect directly to SQL Server/PostgreSQL. Instead, it leverages the existing Grafana Data Source Query API (POST /api/ds/query) to execute pre-defined SQL. The browser interacts only with the local HTTP server. All database work is performed by Grafana against its configured datasources, while the local service handles orchestration, parallelism, caching, aggregation, error handling and UI-ready transformation.

# 5. High-Level Architecture

Figure 5.1 — High-Level System Architecture

┌───────────────┐        ┌────────────────────────┐        ┌────────────────────┐

│   Browser     │  HTTP  │   Local HTTP Server    │        │  DashboardService  │

│ (LAN Users)   │───────▶│  dashboard/server.py   │───────▶│    CommChecker     │

└───────────────┘        └────────────────────────┘        └─────────┬──────────┘

│

▼

┌────────────────────┐

│   GrafanaClient    │

└─────────┬──────────┘

│ POST /api/ds/query

▼

┌────────────────────┐

│      Grafana       │

└────┬──────────┬────┘

│          │

▼          ▼

┌───────────┐  ┌─────────────┐

│ SQL Server│  │ PostgreSQL  │

│ (Device)  │  │ (Results)   │

└───────────┘  └─────────────┘

Key Points:

Single entry point: run.py

Mediated access: Grafana is the gateway to all databases

Read-only: all queries are SELECT statements

Isolated: one failing CLR/query does not impact others

Cached: browsers never trigger direct database work

# 6. Component Architecture

| File / Module | Role | Details |
| --- | --- | --- |
| run.py | Bootstrapper | CLI parsing, config load, auth resolution, demo startup, service init, server start, browser launch |
| dashboard/grafana.py | Transport | Builds /api/ds/query payload, urllib transport, frames→rows parsing, TLS/auth/timeout handling |
| dashboard/queries.py | Query Repository | All SQL builders for MSSQL & PostgreSQL, date validation, escaping |
| dashboard/service.py | Orchestrator & Cache | Background refresher, TTLs, thread pool, on-demand cache, totals, attribute explorer |
| dashboard/commcheck.py | Bulk Verifier | Serial validation, chunking, parallel CLR checks, classification |
| dashboard/server.py | HTTP Layer | REST endpoints, static serving, JSON responses, error mapping |
| dashboard/demo_grafana.py | Mock Provider | Simulated Grafana for --demo and tests |
| static/* | Presentation | index.html, app.js, style.css, bundled vendor charting |
| diagnose.py | Troubleshooter | Per-CLR/profile timing and raw error reporting |

# 7. Database Architecture (Complete)

The application relies on two distinct database engines, each exposed via separate Grafana datasources per CLR. No cross-database joins occur inside SQL; correlation is performed at the application layer.

## 7.1 SQL Server (Device Master) — mssql_uid per CLR

| Table | Purpose | Key Columns | Usage |
| --- | --- | --- | --- |
| Device | Master meter/device table | Pkid, SerialNo, IsTemplate, TemplateFk | Inventory counts; RF/GPRS; existence for comm-check |
| DeviceAttr | Device attribute values | DeviceFk, AttrKeyFk, AttrVal | ConnectUsing, metering mode, Last Communication values |
| AttrKey | Attribute dictionary | Pkid, AttrName | Maps AttrKeyFk to human-readable names |
| MeasPoint | Measuring point per device | Pkid, DeviceFk | Bridge Device ↔ MeasPointResType |
| MeasPointResType | Expected reporting mapping | Pkid, MeasPointFk, ResultTypeFk | Defines which ResultTypeFk a meter is expected to report |

Notes:

ConnectUsing = 'NetworkUdp' → classified as RF.

Meters with TemplateFk IS NOT NULL and no NetworkUdp attribute → classified as GPRS.

Device filter everywhere: SerialNo LIKE 'U%' AND IsTemplate = 0.

Attribute values normalized via COALESCE(NULLIF(LTRIM(RTRIM(CAST(AttrVal AS NVARCHAR(200)))), ''), '(blank)').

## 7.2 PostgreSQL (Measurement Results) — postgres_uid per CLR

| Table | Purpose | Key Columns | Usage |
| --- | --- | --- | --- |
| dbo.result_24h_double | Midnight / 24H results | mprt_fk, result_timestamp | Midnight coverage & midnight trend |
| dbo.result_1month_double | Billing / monthly results | mprt_fk, result_timestamp | Billing coverage (month of selected date) |
| dbo.result_30m_double | Load Profile (30-min) | mprt_fk, result_timestamp | Load profile coverage; hourly distribution |
| dbo.result_0s_double | Instant results | mprt_fk, result_timestamp | Instant data reception |
| dbo.mssql_measpointrestype | Cross-link to expected mapping | pkid, resulttypefk | Join r.mprt_fk = m.pkid to enforce ResultTypeFk |

Common join pattern:

FROM dbo.<result_table> r
JOIN dbo.mssql_measpointrestype m ON m.pkid = r.mprt_fk
WHERE m.resulttypefk = <ResultTypeFk> AND <time_condition>

Time conditions:

| Profile | Today | Past date D |
| --- | --- | --- |
| midnight / load_profile / instant | r.result_timestamp >= CURRENT_DATE | r.result_timestamp >= DATE 'D' AND < DATE 'D' + 1 |
| billing | r.result_timestamp = date_trunc('month', now()) | r.result_timestamp = date_trunc('month', DATE 'D') |

Counting rule: COUNT(DISTINCT r.mprt_fk) — distinct measuring points reporting for that profile/date.

## 7.3 Cross-CLR Mapping (config.json)

| CLR | MSSQL UID | PostgreSQL UID | Result Types (M / B / LP / I) | Metering Mode Key | Last-Comm Keys |
| --- | --- | --- | --- | --- | --- |
| CLR1 | ben8it90cadq8d | ben8jdqkzxs74b | 1605 / 1376 / 1722 / 1635 | 1791 | 2027, 1872, 1728 |
| CLR2 | ben8t031ounswe | aen8rcvlaz7r4c | 2607 / 2378 / 2724 / 2637 | 592 | 873, 751, 645 |
| CLR3 | een8uf46yes5cf | cen8s12bs9fr4c | 715 / 486 / 832 / 745 | 684 | 800, 710, 606 |
| CLR4 | cen8tob820ydcc | ben5kzaupn30ge | 4197 / 3968 / 4314 / 4227 | 748 | 928, 779, 674 |

# 8. End-to-End Data Flow

Figure 8.1 — End-to-End Data Flow

run.py

│  load config.json + ENV auth

▼

GrafanaClient / DashboardService / CommChecker

│                                    │

├── service.start() ──▶ background refresher thread

│                                    │

│                          per section/CLR fetch (parallel)

│                                    ▼

│                          GrafanaClient.query()

│                                    │ POST /api/ds/query

│                                    ▼

│                          Grafana → SQL Server / PostgreSQL

│                                    ▼

│                          frames → rows → cache (_Section)

│

└── serve() ──▶ ThreadingHTTPServer

│

▼

Browser (index.html + app.js)

│ GET /api/view?date=...&days=...

▼

service.view() → JSON snapshot

# 9. Detailed Application Flow

## 9.1 Startup Flow

Figure 9.1 — Startup Flow

python run.py

│

├─ argparse: --config, --host, --port, --demo, --no-browser

├─ read config.json

├─ auth: GRAFANA_TOKEN > GRAFANA_BASIC_AUTH > config.api_token

├─ if --demo: start_demo() → fake Grafana

├─ GrafanaClient(base_url, token, basic, verify, timeout)

├─ DashboardService(client, cfg).start()   # background thread

├─ CommChecker(client, cfg)

├─ serve(service, checker, host, port)

└─ open browser + httpd.serve_forever()

## 9.2 Background Refresh Flow (Live Data)

Figure 9.2 — Background Refresh Flow

_loop() every ~2s

│

├─ for each section:

│     ├─ if pending → skip

│     ├─ wait = 60s if errors else section TTL

│     ├─ if now - last_attempt < wait → skip

│     └─ mark pending=all CLRs; submit _fetch(sec, clr) to pool

│

└─ _fetch(sec, clr):

├─ build query {ref_id, ds_type, ds_uid, sql}

├─ client.query([q])

├─ on success: store rows, updated=now, clear error

└─ on failure: store error, keep last good rows

Default TTLs (seconds):

| Section | TTL | Datasource |
| --- | --- | --- |
| devices | 300 | MSSQL |
| expected | 600 | MSSQL |
| midnight | 120 | PostgreSQL |
| billing | 300 | PostgreSQL |
| load_profile | 180 | PostgreSQL |
| instant | 180 | PostgreSQL |
| metering_mode | 900 | MSSQL |
| lp_hourly | 900 | PostgreSQL |

## 9.3 Historical View Flow (Past Date / Trend)

Figure 9.3 — Historical View Flow

GET /api/view?date=YYYY-MM-DD&days=7|14|30&refresh=0|1

│

├─ _parse_day: format, not future, <= 366 days back

├─ is_today = (date == today)

├─ specs: always (trend) ; if not today add

│         midnight, billing, load_profile, instant, lp_hourly

├─ _ensure(specs, refresh):

│     ├─ prune on-demand cache if > 3000 entries

│     ├─ mark missing/expired entries pending

│     └─ submit _od_fetch to od_pool (2 workers)

├─ merge live sections or on-demand entries per CLR

├─ compute coverage, lp_hourly, trend, metering_mode

└─ _totals() + status → JSON snapshot

On-demand TTL: 300 s (age 0), 1800 s (age 1), 21600 s (age >= 2); 60 s retry on error.

## 9.4 Communication Check Flow

Figure 9.4 — Communication Check Flow

POST /api/comm-check {serials}

│

├─ parse_serials: split ws/comma/;/|/tab

│     regex ^[A-Za-z0-9_-]{1,40}$ ; dedupe (case-insensitive)

├─ if none valid → ValueError

├─ if > max_serials (20000) → ValueError

├─ chunk into groups of 400

├─ create job (<= 3 running) ; purge > 1h old

├─ thread: for each chunk × CLR → Q.last_comm_sql

│     per_clr[clr][SERIAL] = (last_comm, found_attr)

│     failed[clr] = error

├─ classify():

│     found_attr → Communication (latest last_comm)

│     else if failed CLR → Unknown

│     else if exists but no attr → Non-communication (no value)

│     else → Non-communication (not found)

├─ summarize(): totals, by_clr, age_buckets, invalid/dups

└─ job.status = done ; GET /api/comm-job?id=… polls

## 9.5 Attribute Explorer Flow

Figure 9.5 — Attribute Explorer Flow

GET /api/attr-names

├─ cache 'names' (TTL 3600 s)

├─ query all CLRs: SELECT DISTINCT AttrName FROM AttrKey

└─ union names (case-insensitive sort)

GET /api/attr?name=…

├─ validate against attr_names()

├─ cache 'attr:<name>' (TTL 300 s)

├─ per CLR: TOP 200 values with COUNT(DISTINCT SerialNo)

└─ return {name, clrs, errors, truncated, generated_at}

# 10. API Reference

| Method | Path | Query / Body | Purpose |
| --- | --- | --- | --- |
| GET | / or /* | — | Serve static web assets (index.html, app.js, style.css, vendor) |
| GET | /api/view or /api/snapshot | date, days=7¦14¦30, refresh=0¦1 | Full dashboard snapshot |
| GET | /api/attr-names | — | Distinct attribute names from all CLRs |
| GET | /api/attr | name=<AttrName> | Attribute distribution per CLR (TOP 200) |
| POST | /api/comm-check | {"serials":"..."} | Start bulk communication check |
| GET | /api/comm-job | id=<job_id> | Poll job status/result |
| GET | /api/health | — | Liveness probe |

Error mapping:

ValueError → 400 Bad Request

GrafanaError → 502 Bad Gateway

Other Exception → 500 Internal Server Error

Unknown /api/* → 404 JSON

Path traversal or missing static → 404

# 11. Caching, Resiliency & Performance

## 11.1 Caching Strategy

| Layer | Scope | TTL / Behavior |
| --- | --- | --- |
| _Section.data | Per (section, CLR) live | Section TTL; 60 s retry on error; last good value kept |
| self.od | On-demand (kind, day, extra, CLR) | 300 s / 1800 s / 21600 s by age; 60 s on error; pruned at 3000 |
| _attr_cache | names / attr:<name> | 3600 s / 300 s |
| Browser | Static assets & JSON | no-cache for static; no-store for JSON |

## 11.2 Resiliency

Per-query isolation: each (section, CLR) and (chunk, CLR) is independent

Graceful degradation: previous rows retained; error + elapsed recorded

Parallelism limits: max_parallel_queries (default 4); od_pool = 2

Timeouts: request_timeout_seconds (default 240)

Auth fallback: ENV tokens override config

TLS flexibility: verify_tls=false for internal certificates

Comm-check safety: failures yield Unknown, never false Non-communication

Input hardening: regex validation, chunking, 2 MB body cap, 3-job limit

## 11.3 Performance Notes

Background-first: DB work off the request path

Distinct counting matches Grafana semantics

Attribute distributions limited to TOP 200 per CLR

Chunked comm-check (400) across up to 4 CLRs

ThreadingHTTPServer for concurrent browsers

Bounded thread pools avoid resource exhaustion

# 12. Frontend Design

| File | Responsibility |
| --- | --- |
| static/index.html | Layout: title, date controls, tiles, charts, tables, health, definitions |
| static/app.js | Fetch /api/view, render charts/tables, date navigation, refresh, comm-check, CSV export |
| static/style.css | Theming and layout |
| static/vendor/ | Bundled charting assets (no CDN) |

URL-synced date: ?date=YYYY-MM-DD

Total (all CLRs) series with partial indicator (X of Y CLRs)

Legend click toggles Total series

Health table shows failures, timing and age

Explicit comm-check reasons (typo vs no last-comm vs unknown)

# 13. Configuration Reference (config.json)

{
  "grafana_url": "https://grafana.ugvcl.net:3000",
  "verify_tls": false,
  "api_token": "",
  "request_timeout_seconds": 240,
  "refresh_seconds": { "devices": 300, "expected": 600, "midnight": 120,
                       "billing": 300, "load_profile": 180, "instant": 180,
                       "lp_hourly": 900, "metering_mode": 900 },
  "clrs": { "CLR1": { ... }, "CLR2": { ... }, "CLR3": { ... }, "CLR4": { ... } },
  "max_parallel_queries": 4,
  "comm_check": { "chunk_size": 400, "max_serials": 20000 },
  "app_title": "UGVCL Smart Meter Live Monitor",
  "app_subtitle": "Data received, communication and device inventory across CLR-1 to CLR-4"
}

| Key | Type | Required | Notes |
| --- | --- | --- | --- |
| grafana_url | string | Yes | Base URL of Grafana |
| verify_tls | bool | Yes | false for internal/self-signed certs |
| api_token | string | No | Prefer GRAFANA_TOKEN env var |
| request_timeout_seconds | int | No | urlopen timeout (default 60) |
| refresh_seconds | object | No | Per-section TTL overrides |
| clrs.<CLR>.* | object | Yes | Datasource UIDs, result/attr keys |
| max_parallel_queries | int | No | Thread pool size (default 4) |
| comm_check.chunk_size | int | No | Serial batch size (default 400) |
| comm_check.max_serials | int | No | Hard limit (default 20000) |
| app_title / app_subtitle | string | No | UI branding |

Auth resolution order: GRAFANA_TOKEN → GRAFANA_BASIC_AUTH → config.api_token.

# 14. Deployment Guide

## 14.1 Prerequisites

Python 3.8+ on the host

Network access from host to grafana_url

Valid Grafana Service Account token (recommended)

Access to SQL Server/PostgreSQL through existing Grafana datasources

## 14.2 Steps

1. Copy the entire ugvcl-live-dashboard/ folder to the target machine.

2. Review config.json (grafana_url, CLR UIDs, result/attribute keys).

3. Set authentication: GRAFANA_TOKEN=<token> (preferred) or GRAFANA_BASIC_AUTH=user:password.

4. Run: python run.py [--host 0.0.0.0] [--port 8050] [--demo] [--no-browser]

5. Open http://127.0.0.1:8050 (or the LAN URL) in a browser.

6. Verify sections load; run python diagnose.py if anything is missing.

## 14.3 Notes

No pip/npm installation is required (standard library only).

The app runs in the foreground; wrap with a process manager for production.

Use --demo for offline validation without any Grafana connectivity.

diagnose.py runs each query once per CLR/profile with timing and raw errors.

# 15. Diagnostics & Troubleshooting

| Symptom | Likely Cause | Action |
| --- | --- | --- |
| Cannot reach Grafana | DNS / firewall / VPN | Test connectivity to grafana_url and port |
| HTTP 401/403 | Auth missing or insufficient | Set GRAFANA_TOKEN or GRAFANA_BASIC_AUTH |
| TLS errors | Internal/self-signed certificate | Set verify_tls=false (with awareness) |
| Slow/timeout queries | Heavy SQL or Grafana load | Run diagnose.py; raise request_timeout_seconds if appropriate |
| Missing numbers for a CLR | Query failure for that CLR | Check health/status and per-section errors |
| Unknown in comm-check | CLR query failed | Inspect job errors; fix datasource/auth for failing CLR |
| Date rejected | Future or > 366 days back | Choose a date within the allowed range |
| Too many serials | Exceeds max_serials | Split the input (limit 20000) |
| Cannot start check | 3 jobs already running | Wait for a job to finish (>1h auto-purged) |

# 16. Security Considerations

| Area | Position |
| --- | --- |
| Network | Intended for internal LAN; 0.0.0.0 shares within office network |
| Authentication | Delegated to Grafana; tokens via ENV, never committed |
| Data Access | Read-only SELECT statements only |
| Input Validation | Serial regex, date validation, body cap, path traversal protection |
| Secrets | api_token may remain empty when ENV is used; tokens never logged |
| TLS | verify_tls=false only for internal legacy certificates |
| Error Disclosure | JSON errors without stack traces to clients |
| Concurrency | Job cap + aging cleanup limit abuse |

# 17. Ready-To-Purpose Checklist

☐  config.json validated (grafana_url, all 4 CLRs, UIDs, result_types, attr keys)

☐  GRAFANA_TOKEN (or basic auth) configured and verified

☐  Offline validation with --demo completed

☐  Live mode verified: tiles, charts, tables populate

☐  Date selector, trend lengths and Refresh behave correctly

☐  Communication check tested (valid, invalid, duplicate, unknown cases)

☐  Attribute explorer returns names and distributions

☐  Health table shows timing and no persistent errors

☐  LAN access confirmed (if --host 0.0.0.0 is used)

☐  Operations team briefed with this document and README.md

# 18. Appendix

## A. Query Reference by Function

| Function | Datasource | Purpose |
| --- | --- | --- |
| devices_sql | MSSQL | Total / RF / GPRS device counts |
| expected_sql | MSSQL | Expected meters per profile via MeasPointResType |
| metering_mode_sql / attr_by_key_sql | MSSQL | Metering mode distribution |
| attr_names_sql / attr_by_name_sql | MSSQL | Attribute explorer |
| metric_sql / today_sql | PostgreSQL | Distinct reporting meters per profile/date |
| lp_hourly_sql | PostgreSQL | Hourly load-profile reporting meters |
| midnight_trend_sql | PostgreSQL | Midnight trend over N days |
| last_comm_sql | MSSQL | Last Communication lookup for comm-check |

## B. Consolidated System Flowchart

Figure 18.1 — Consolidated System Flowchart

[Launch run.py]

│

▼

[Load config + ENV auth]

│

├──────────────┐

▼              ▼

[Background      [HTTP server]

refresher]           │

│               ▼

│         [Dashboard UI]

│               │

│               ▼

│        [Poll /api/view]

│               │

▼               ▼

[Query Grafana] ◀── [Cache miss → on-demand]

│

▼

[SQL Server + PostgreSQL]

│

▼

[Normalize → aggregate → totals]

│

▼

[Render tiles / charts / tables]

## C. File Structure

ugvcl-live-dashboard/
├── config.json
├── diagnose.py
├── README.md
├── run.py
├── dashboard/
│   ├── __init__.py
│   ├── commcheck.py
│   ├── demo_grafana.py
│   ├── grafana.py
│   ├── queries.py
│   ├── server.py
│   └── service.py
├── static/
│   ├── app.js
│   ├── index.html
│   ├── style.css
│   └── vendor/
└── tests/
    └── test_dashboard.py

— End of Document —